Good — let me lay out the whole Celik/Richardson chain symbol by symbol, with the formula, its units, a plain-English meaning, and your actual thesis value so each one is concrete. First, the indexing convention, because it's the thing people trip on:

Grid numbering: subscript 1 = finest, 2 = medium, 3 = coarsest (so h₁ < h₂ < h₃). The ratios are therefore written r₂₁, r₃₂ — "coarser over finer" — not r₁₂. (You wrote r12; the standard name for the medium-to-fine ratio is r₂₁.)

1. Representative cell size, h

$$h = \left[\frac{1}{N}\sum_{i=1}^{N}\Delta V_i\right]^{1/3}$$

- N = total number of cells; ΔVᵢ = volume of cell i.

- Units: metres (m) — it's a characteristic edge length of a "typical" cell.

- Meaning: one number that stands for how fine the grid is. The cube root makes it a length in 3-D. Because your domain volume is fixed, this reduces to h ∝ N^(−1/3): quadruple the cells, cell size drops by 4^(1/3) ≈ 1.59.

2. Refinement ratio, r

$$r_{21}=\frac{h_2}{h_1}, \qquad r_{32}=\frac{h_3}{h_2}$$

- Units: dimensionless (a length ÷ a length).

- Meaning: how much coarser one grid is than the next finer one; always > 1. Celik recommends ≥ 1.3 so the grids are "meaningfully different."

- Your values: r₂₁ = 1.208, r₃₂ = 1.413.

3. Solution differences, ε

$$\varepsilon_{21}=\phi_2-\phi_1, \qquad \varepsilon_{32}=\phi_3-\phi_2$$

- φₖ = the quantity you're tracking, computed on grid k. Here φ = ΔP, so units = mPa (and ε has the same units).

- Meaning: how much the answer changed when you refined the grid. If ε shrinks as you refine, the solution is converging. The sign matters: same sign for both = monotonic convergence; opposite signs = oscillatory.

- Your values: ε₂₁ = 11.3736 − 11.4099 = −0.0363 mPa; ε₃₂ = 11.2522 − 11.3736 = −0.1214 mPa (both negative → monotonic).

4. Apparent order of accuracy, p

$$p=\frac{1}{\ln r_{21}}\Big|\ln\left|\tfrac{\varepsilon_{32}}{\varepsilon_{21}}\right| + q(p)\Big|, \qquad q(p)=\ln!\left(\frac{r_{21}^{,p}-s}{r_{32}^{,p}-s}\right), \quad s=\mathrm{sign}!\left(\tfrac{\varepsilon_{32}}{\varepsilon_{21}}\right)$$

- Units: dimensionless.

- Meaning: the observed order at which your error falls as the grid refines — ideally it matches the scheme's theoretical order (2 for a second-order method). p appears on both sides, so it's solved by iteration. The q(p) term is only needed because your two ratios differ; if r₂₁ = r₃₂ it vanishes and p = ln|ε₃₂/ε₂₁| / ln r.

- Your value: p = 2.20 (near the theoretical 2 — a healthy sign).

5. Richardson extrapolated value, φ_ext

$$\phi_{\mathrm{ext}}^{21}=\frac{r_{21}^{,p},\phi_1-\phi_2}{r_{21}^{,p}-1}$$

- Units: same as φ, so mPa.

- Meaning: the estimate of the "perfect grid" answer — what ΔP would be at zero cell size. It's obtained by projecting the power-law trend to h → 0. This is the actual Richardson extrapolation step.

- Your value: φ_ext = 11.48 mPa.

6. Approximate relative error, e_a

$$e_a^{21}=\left|\frac{\phi_1-\phi_2}{\phi_1}\right|$$

- Units: dimensionless (report as %).

- Meaning: the fractional change between the two finest grids — a raw "how much is still moving" number, before any safety margin.

- Your value: e_a = 0.318%.

7. Grid Convergence Index, GCI (the headline result)

$$\mathrm{GCI}{\mathrm{fine}}^{21}=\frac{F_s,e_a^{21}}{r{21}^{,p}-1}$$

- F_s = safety factor = 1.25 (for three or more grids); units: dimensionless (%).

- Meaning: the conservative uncertainty band on your fine-grid result due to discretisation. Read it as "the true answer is within ±GCI% of the fine-grid value, at ~95% confidence." It's the Richardson error estimate, divided by the (rᵖ − 1) Richardson factor, padded by the 1.25 margin.

- Your values: GCI₂₁ = 0.77% (fine), and by the same formula with the coarser pair, GCI₃₂ = 1.18% (medium/production mesh).

8. Asymptotic-range check

$$\frac{\mathrm{GCI}{32}}{r{21}^{,p},\mathrm{GCI}_{21}} \approx 1$$

- Units: dimensionless.

- Meaning: the sanity test that the whole Richardson assumption (single power-law error) actually holds. If the GCIs shrink between grid levels at exactly the rate rᵖ predicts, the ratio comes out ≈ 1 and you're in the "asymptotic range" where all the above is valid. If it's far from 1, the extrapolation is untrustworthy.

- Your value: 1.003 — essentially perfect, which is what licenses everything else.

How to read the chain in one sentence: measure how much ΔP changes between grids (ε), turn that into the observed order p and the zero-grid answer φ_ext (Richardson), express the leftover error as a fraction (e_a), pad it with a 1.25 safety factor to get the uncertainty band (GCI ≈ 1.2% on your production mesh), and confirm the whole procedure is valid with the ≈1 asymptotic check.

If it helps, I can add a compact "definition of terms" box to Appendix A — a small table listing each symbol, its formula, and units — so a reader doesn't have to reconstruct them. Want that? It'd cost about a third of a page and use no new references.